SECTION 02790

ATHLETIC AND RECREATIONAL SURFACING

PART 1 GENERAL

  - SECTION INCLUDES

- Resilient interlocking fitness flooring

  - RELATED SECTIONS

- Section 02300 – Earthwork:  Asphalt subsurface.

- Section 03300 – Cast-in-Place:  Concrete subsurface.

  - REFERENCES

- ASTM D412 – Standard Test Methods for Vulcanized Rubber and                        Thermoplastic Elastomers.

- ASTM D624 – Standard Test Method for Tear Strength of Conventional Vulcanized Rubber and Thermoplastic Elastomers.

- ASTM E303 – Standard Test Method for Measuring Surface Frictional Properties Using the British Pendulum Tester.

- ASTM D3676 – Standard Specification for Rubber Cellular Cushion Used for Carpet or Rug Underlay.

- ASTM C501 – Standard Test Method for Relative Resistance to Wear of Unglazed Ceramic Tile by the Taber Abraser.

  - SUBMITTALS

- Submit under provisions of Section 01300

B.   Product Data:

1. Installation and Maintenance Instructions.

2. Storage and Handling Instructions

3. Product Warranty

- Selection Samples:  Submit one set of color chips representing         manufacturers full range of available colors.

- Verification Samples: Submit one sample, minimum size 6 inches square

- Installers Project References:

- Submit list of successfully completed projects.

- Include project name and location, name of architect, and type and quantity of surfacing installed.

  - QUALITY ASSURANCE

- Manufacturers Qualifications:  Manufacturer shall meet the following requirements

- Continuously engaged in manufacturing of surfacing products of similar type to that specified.

- Installers Qualifications:

- Trained and certified by the manufacturer.

- Approved by the manufacturer.

1.6   DELIVERY, STORAGE AND HANDLING

- Delivery:  Deliver materials to site in manufacturer’s original, unopened containers and packaging, with labels clearly identifying product name and manufacturer.

- Storage:

- Store materials in accordance with manufacturer’s instructions.

- Store materials to be installed indoors at the ambient temperature of the installation location for a minimum of 48 hours before installation.

- Protect materials from direct sunlight before installation.

- Store adhesive indoors at the ambient temperature of the installation location.

  - PROJECT CONDITIONS

- Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results.  Do not install products under environmental conditions outside manufacturer’s absolute limits.

- Air Temperature for Interior Installations:  Ensure air temperature is the same temperature as anticipated during normal operation.

  - WARRANTY

- Materials and Workmanship:  Aveer Sport has a limited 10 year warranty per the warranty schedule.

PART 2 PRODUCTS

  - MANUFACTURERS

- Acceptable Manufacturer:  Rubberecycle LLC., 1985 Rutgers University Blvd.,  Lakewood, NJ 08701.  Toll Free: 888.436.6846  Fax: 732.370.4247.

- Requests for substitutions will be considered in accordance with provisions of section 01600.

  - PROPRIETARY PRODUCTS

- Tiles: Aveer Sport

- Description: Vulcanized rubber, interlocking fitness flooring

- Material:  Vulcanized SBR and TPE rubber

- Seams: U-Channel interlocking configuration

- Edges:  Zero radius edge

- Bottom:  Reversed prisim pedestal configuration

- Top:  Diamond pattern texture

- Size: 24 1/16t”X24 1/16t”X1” thick

- Minimum Weight: 20 Lbs.

- Colors: Black

- Test Results

- ASTM E303

  - Dry: 63

  - Wet: 42

- ASTM D412

  - Tensile Strength: 2.68 Mpa

  - Elongation at Break: 110%

- ASTM D624

  - Tear Strength: 14.0 kN/m

  - Peak Tear Load: 175 (N)

- ASTM D3676

  - 1534 kg/m3

- ASTM C501

  - Wear index: 52

- Accessories

- Reducers

- Prefabricated ADA compliant ramped edging

- Material:  Same as tiles.

- Adhesive:  Furnished by manufacturer

PART 3 EXECUTION

3.1   EXAMINATION

- Examine areas to receive fitness flooring tiles.  Notify architect if areas are not acceptable.  Do not begin installation until unacceptable conditions have been corrected.

3.2   PREPARATION

- Prepare subsurface in accordance with manufacturer’s instructions.

3.3   INSTALLATION

- Install fitness flooring in accordance with manufacturers instructions and drawings.

- Ensure prepared subsurface is dry and clean.

- Layout tile surface in accordance with manufacturer’s instructions and drawings

- Apply Adhesive in accordance with manufacturers instructions

Installation to be completed by a factory trained and authorized installer

3.4    CLEANING

- Clean tiles in accordance with manufacturer’s instructions

3.5   PROTECTION

- Protect completed flooring from damage during construction.

END OF SECTION